| Product: | ## RTL8712(8188_8191_8192SU) USB Software Package - Linux Driver |
| --- | --- |
| Version: | V2.6.6.0.20110401 |

Release History:

| V1.0.0001 – 2009/01/21 | - First Released - only supports USB interface - add features about 802.11n draft 2.0 - RX-AMSDU (4K, 8K) - currently only supports 20MHz BW |
| --- | --- |
| V2.0.0001 – 2009/4/27 | - WiFi 11n-Certificated - Supported 20/40 MHz BW Switching - Supported Fedora 10 |
| V2.6.0001 – 2009/9/30 | - Add Rx_BH - Add Tx_BH - Fixed system hung up issue when cancel rx timer. |
| V2.6.0005 – 2009/10/13 | - put usb rx aggregation to the rx_bh - support Rx TCP_CSUM_OFFLOAD - fixed ping issue on DMP Platform with D-LINK DIR655 AP - To report encryption_cap. When wpa_supplicant & Network Management require |
| V2.6.0005 – 2009/11/12 | - fixed ad-hoc issue when using Network Manager - update fw to fixed long run testing issue |
| V2.6.0005.p1 – 2009/11/12 | 1. Fixed the power saving issue. |
| V2.6.0005.p2 – 2009/11/12 | 1. support high power mode |
| V2.6.0005.p3 – 2009/11/12 | - Added a script to clean the r8192s_usb driver for avoiding the conflict that 8712u and r8192s_usb drivers are running at the same time. - Updated the thread_enter function for some SDIO bugs. |
| V2.6.0005.p4 – 2009/11/12 | - Added two VID/PIDs into supporting list. |
| V2.6.0005.20100105 | 1. Added two VID/PIDs into supporting list. |
| V2.6.0005.20100106 | 1. Fixed the compiling error under kernel 2.6.18. ( Fedora 6 ) |
| V2.6.0005.20100121 | 1. Added one VID/PID into supporting list. |
| V2.6.0005.20100126 | 1. Added one VID/PID into supporting list. |
| V2.6.0006.20100202 | - Updated VID/PID table - Refine the Tx/H2C mechanism on USB single or multiple 64/512 bytes case. - Added the Software LED control - Fixed the ESSID bug under 2.6.19.2 - Fixed the ad-hoc bug when switching the security - Improved the interoperability and dynamic algorithm |
| V2.6.0006.20100226 | - Fixed the crash issue on mstar platform. - Correct the signal strength. - Support the video mode. - Improved the interoperability and dynamic algorithm. - Fixed the adhoc master problem. |
| V2.6.0006.20100409 | - Fixed the big endian issue for software WEP/TKIP encryption/decryption. - Fixed the crash issue when users unplug the usb dongle or remove the driver from system. - Improve the AMSDU throughput. - Added the error handling in site survey function. - Corrected the LED behavior. - Improve interoperability. |
| V2.6.0006.20100430 | - Added the version.h file in the include folder. - Added more PID/VID into supporting list. - Fixed the big endian issue in WPA-PSK TKIP mode with Linksys WRH54G AP. - Improve the interoperability for high performance APs. - Fixed the AP Tx fragment issue. |
| V2.6.0006.20100507 | - Supported using the mac address which users specify. |
| V2.6.0006.20100511 | - Supported the kernel 2.6.32. - Improved the throughput on specific platforms. |
| V2.6.0006.20100601 | - Fixed some WPS IOT issue with Corega AP and Netgear AP. - Improved the throughput on specific platforms. - Fixed potential redefinition error with kernel 2.6.18. - Fixed the bug which it is hard to get IP from Netgear AP. - Driver will show up the driver version information when it is inserted into the system. - Improved the keep alive function with Netgear WNR2000 AP. |
| V2.6.6.0.20100617 | - Rename the version rule to v6.0 - Fixed the IOT connection issue with Netgear 3700 - Fixed the potential crash issue when connecting to the Linksys 350N. |
| V2.6.6.0.20100623 | - Driver will use the bg mode to connect to the WEP mode AP. |
| V2.6.6.0.20100625 | - Updated the mechanism to compute the link quality. - Use the percentage format for signal strength and link quality display. |
| V2.6.6.0.20100823 | - Fixed the connection problem when the adhoc SSID is the same as the existing AP’s SSID. - The firmware will send the de-authentication to AP if the WiFi dongle thinks the AP had disappeared. - The firmware will send more NULL data packet to check the current network (AP) is alive or not. |
| V2.6.6.0.20100930 | - Fixed the BitRate information when using the iwconfig. - Added the Realtek GPL license. |
| V2.6.6.0.20101111 | - Added Android support - Updated VID/PID table |
| V2.6.6.0.20101224 | - Update firmware to 516 - Use stack instead of heap for usbctrl_vendorreq's buffer use - Modify Makefile and add Kconfig to Support builtin & module build under kernel tree - Fix the Android private wifi ioctl SCAN-ACTIVE, SCAN-PASSIVE - Fix the connect/disconnect looping problem on Android. |
| V2.6.6.0.20110401 | - Updated the wpa_cli_with_wpa_supplicant_20110401.doc - Updated the optimize parameter of Makfile of wpa_supplicant - Reduce the Rx buffer size to avoid the long run WiFi issue on mstar platform. - Fixed the compile error on kernel 2.6.37. - Added the WPS cancel function. - Added the channel plan. |

Description:

Realtek provides the solution for WLAN in hardware and software. The software package could be adopted in Realtek demo-board in the Linux platform.

Software Version:

RTL8712 USB Linux Driver Version: v2.6.6.0.20110401

SVN1497

Firmware Version:

Firmware is embedded in the RTL8712 USB driver.

The firmware version of this release is SVN516

Software Package Included:

a. Readme.txt

b. Release Notes

c. Linux driver

d. wpa_supplicant - wpa_supplicant-0.6.9_wps_patch_20110401.zip

e. document - quick_installation_guide.ppt

f. document - driver programming guide.doc

g. document - wireless tool porting guide.doc

h. document - wpa_cli_with_wpa_supplicant_20110401.doc

i. document - sample code for hardware wps PBC

j. document - HowTo support more VidPids.doc

k. document - HowTo support new platform(including Android).doc

l. document - HowTo build driver under kernel tree.doc

m. wpa.conf - the config file for wpa_supplicant launch

Hardware Supported:

Realtek RTL8712/8188/8191/8192SU USB demo board:

- WIFI – SoC, including a CPU, one 802.11b/g/n MAC/Baseband/RF controller with PA for the USB interface.

Platform Supported:

- USB : LINUX (kernel 2.6.18 ~ 2.6.37)

Software Package Feature:

- IEEE 802.11 b/g/n WiFi compliant

- 802.1x, WEP, WPA TKIP and WPA2 AES/Mixed mode for PSK and TLS (Radius)

- Wireless Auto-channel selection

- WPS - PIN and PBC Methods

- IEEE 802.11b/g/n client mode

  - Support Ad-hoc and infrastructure mode with roaming capability

  - Support wireless security for WEP, WPA TKIP and WPA2 AES PSK

  - Support site survey scan and manual connect

  - Support WPA/WPA2 TLS client

To-Do List:

- To Add Soft AP Mode